Project 1 Report

The Quine-McCluskey

Algorithm in C++

Mohammad Dawood (SID:900242164)

Michael Reda (SID: 900242473)

Ali Mohamed (SID: 900246190)

• The Program Design, including any used Data Structures and Algorithms:

The program is designed to automate logic minimization using the Quine-McCluskey algorithm in C++. The code is completely implemented in one .cpp file and can be run directly from the terminal using the QM class. The design is divided into fundamental phases, which correspond to the two parts of the QM method:

- Prime Implicant Generation: A tabular method (matching () function) to find all the Prime Implicants (PIs) of the given function.

- A covering algorithm (PITable () and solveRecursive ()) to find all minimal Sum-of-Products.

Data Structures used include:

### A struct implicant, which contains:

  - set<int> mins: to store the set of original minterms that the implicant covers.

  - string binary: to hold the binary representation of the implicant.

  - bool marked: A flag that indicates whether the implicant is matched or not.

  - int id: An ID to manage the PI chart in the recursive solving phase.

### The main class QM, which contains:

  - int numvar: to store the number of Boolean variables.

  - bool terms_are_ok: A flag for input validation.

  - vector<int> minterms: A vector holding the input minterms to be covered.

  - vector<int> Maxterms: A vector for any input maxterms (if they exist).

  - vector<int> dontcares: A vector for the "don't care" input terms.

  - vector<vector<implicant>> columns: A 2D vector representing the implicant grouping columns for the tabulation method.

  - vector<implicant> matched: A vector to store all Prime Implicants (PIs) found.

  - vector<implicant> finalEPIs: A vector to store the identified Essential Prime Implicants (EPIs).

  - vector<vector<implicant>> finalSol: A 2D vector to store all possible minimal solutions (each solution is a vector of implicants).

- Sets, pairs, and strings are also included in different parts of the code.

The Algorithms used are:

- Brute Force: which is used in the matching() function to match the minterms with each other before forming the prime implicants chart.

- Recursive Search (Backtracking) Algorithm: to find all the possible final minimal solutions.

- Selection Sort: used in matching to maintain order in the columns.

• Challenges:

- Understanding the Quine–McCluskey algorithm: We had the initial challenge of understanding the Quine-McCluskey algorithm itself to be able to replicate it correctly.

- Choosing suitable data structures: We then had the challenge of determining the data structures to use since we were not sure of what entities would be responsible for what exactly.

- File paths handling: due to the different platforms and IDEs we were using, we initially faced some problems in feeding the correct paths to the program. This issue remained until it was resolved in the file system menu at the end of the project.

- Optimization difficulties: We then had some troubles in optimization (to be discussed in the problems and remaining issues).

- Printing all the minimal possible solutions: There is also the challenge of printing all of the minimal possible solutions, where we thought at first that using recursion would be complicated, but after understanding how it works, we implemented it in the solveRecursive() function and made sure that it works after testing.

• Testing:

- Edge-case and error-input coverage
 As for testing, we, on a very fundamental level, tried to cover all the edge cases and error inputs that would happen in the reading process (during the initial coding of the readtxt() function).

- Cases considered
 We considered cases such as

  - empty files

  - not proper input formats

  - wrong minterms/dontcares

  - menu validations

  - directory/files existence

  - every core function functionality, using flags and printing helpers

- Used test cases
 The following cases were used:

| Test File | Content | Reason |
| --- | --- | --- |
| Test1.txt | 1 m1 | Simple 1-variable case |
| Test2.txt | 4 m4 | 4-variables, but only one minterm |
| Test3.txt | 4 m0 , m2 ,m8 ,m10 d6 , d14 | Normal 4-variable case. |
| Test4.txt | 4 M1,M3,M4,M5 ,M6 ,M7 ,M9 ,M11 ,M12 ,M13 ,M14 ,M15 d6 ,d14 | 4-variable case equivalent to Test3 case, but in maxterms. |
| Test5.txt | 5 m0, m2, m3, m5, m7, m9, m11, m13, m14, m16, m18, m24, m26, m28, m30 | 5-variable case that yields two possible solutions. |
| Test6.txt | 4 m0 m2 m3 m4 m5 m7 m8 m10 m11 m15 d12 d13 | 4-variable case that yields two possible solutions. |
| Test7.txt | 13 m45 m128 m456 m789 m1023 m1456 m1823 m2047 m2456 m2871 m3124 m3456 m3891 m4123 m4567 m4891 d200 d800 d1200 d2000 d3000 | 13-variable case. (We started to increase the complexity) |
| Test8.txt | 16 m125 m287 m459 m612 m789 m1023 m1456 m1823 m2047 m2456 m2871 m3124 m3456 m3891 m4123 m4567 m4891 m5123 m5567 m5891 m6123 m6456 | 16-variable case. (more complex) |
| Test9.txt | 18 m1250 m2870 m4590 m6120 m7890 m10230 m14560 m18230 m20470 m24560 m28710 m31240 m34560 m38910 m41230 m45670 m48910 m51230 m55670 m58910 m61230 m64560 m68910 m71230 d10000 d20000 d30000 d40000 d50000 | 18-variable case. |
| Test10.txt | 20 m125 m287 m459 m612 m789 m1023 m1456 m1823 m2047 m2456 m2871 m3124 m3456 m3891 m4123 m4567 m4891 m5123 m5567 m5891 m6123 m6456 m6891 m7123 m7456 m7891 m8123 m8456 m8891 m9123 m9567 m9891 m10123 m10456 m10891 m11123 m11456 m11891 m12123 m12456 m12891 m13123 m13456 m13891 m14123 d167 d423 d891 d1234 d1678 d2234 d2678 d3234 d3678 d4234 d4678 d5234 d5678 d6234 d6678 | 20-variable case using both minterms and don’t cares. (This should be the most complex we can achieve.) |

• Instructions to build and use the program:

- The user interacts with the program via the terminal.

- First, the user shall provide the absolute path of the folder containing the input test files.

These test files should be text files and are the main inputs for the application. Each file should have at most 3 lines. The first line contains the number of variables, the second line includes the minterms (indicated by m) or the maxterms (indicated by M) separated by commas, and the third line contains the don’t-care terms separated by commas.

For example, the following could be the content of any Test File. The input specifies a function of 3 variables, 4 minterms (1, 3, 6, and 7), and 2 don’t-care terms (0 and 5):

3

m1,m3,m6,m7

d0,d5

- Next, the program would provide the user with a list of .txt files that were found inside the provided absolute path directory. Next, the program would give the user the option to select one of the found text files or to abort the program.

- Selecting the desired test file, the program would display the following through the terminal in order:

- Details of the chosen test file, such as: number of variables and their names, number of minterms, maxterms (only if provided), and the don’t cares.

- All the found prime implicants after the tabular method (matching()) and before the coverage chart.

- All the possible combinations of prime implicants (solutions) after the coverage charts.

- All the final expressions corresponding to all the possible solutions.

- Verilog modules for all the solutions.

- Afterwards, the menu reappears in the terminal, asking the user to make a choice again.

Note: The implementation of file system handling for the menu (in int main()) allows the application to adapt to existing file changes, meaning the user can change the contents of any test file after each choice; however, the user cannot add or remove test files mid-execution of the application. They should have to stop the app, modify (add or remove) the desired files, then re-run the application; otherwise, the application would not be able to identify the modified files, yet it would not lead to a crash.

• Problems/Remaining Issues in your Program:

There are minor problems in our program:

- Sometimes, the terminal does not fit all of the answers when the possible solutions exceed a certain number.

- In rare cases, the code takes a lot of time to generate the expressions when the number of minterms is too big (or the number of maxterms is too small) and the number of inputs exceeds 13 to 15 inputs; however, it handles all cases of 1-10 inputs efficiently.

- Validations to the contents of the test files are not handled, meaning an input test file that does not follow the specified format may result in unidentified behavior.

• The contributions of each member:

Michael Guirgis:

- Initialized the GitHub repo which includes the contributions of all the team members.

- Participated in prototyping the structure of the QM class with all of the methods and variables.

- Implemented the three methods PITable(), solveRecursive(), and applyDominance() which are responsible for finding all the minimal Sum-of-Products.

- Fixed some minor bugs in the printing functions towards the end of the project.

Mohammad Dawood:

- Participated in prototyping the structure of the QM class with most of the methods and variables.

- Made the initial 5 test cases in the first milestone.

- Implemented the void matching() function responsible for prime implicants generation in the first phase of the algorithm.

- Implemented the following functions:

  - void readtxt()

  - implicant merge(implicant a, implicant b)

  - helper: bool allIsNotMarked()

  - helper: void sortC()

  - void matching()

  - void printMembers()

  - void printAllFinalPIs()

  - void printOutputExp()

  - void runQM()

  - int main() handling for testing/ made the menu and filesystem handling.

Ali Mohamed:

- Participated in prototyping the structure of the QM class with most of the methods and variables.

- Implemented the string IntegerToBinary() that receives two parameters, the number itself and the number of bits that this integer is going to transform to.

- Implemented implicant merg() That merges two implicants and prints ‘-’ in the non-matched bit.

- Implemented void printVerilogModule(); that receives a vector of binary bits for the minimal expression and transforms it into a verilog module.

- Implemented bool isGreyCode which returns true if two implicants have 1 bit difference.